WATER RESOURCES

Version 31, May 2011

Opportunities and challenges

Problems related to water and development

With some 1,200 inhabitants/km2 the delta country Bangladesh is one of the most densely populated regions on earth. Besides the country has a booming economy. These two phenomena are leading to high pressure on land, even within rivers.

Most of the delta is still active with unstable river branches and land accretion in and along rivers and in the Bay of Bengal. Floods are a permanent threat; in a normal year 20% of the country is inundated by river spills and drainage congestions (Bucx et al., 2010). Due to its geophysical setting Bangladesh is frequently confronted by tidal surges and sometimes also by cyclone-induced high storm surges. During the last 48 years nineteen major cyclones hit the coastal area. Much has been done to reduce the number of casualties during tropical storms, e.g. construction of cyclone shelters; however, the risks of economic and infrastructural losses remain significant.

The relative abundance of fresh water in the country due to an average annual rainfall of more than 1.500 mm and water flowing into the delta via the three major river systems of the Ganges, Brahmaputra and Meghna, makes irrigated agriculture and other water based industries possible as well as cheap inland water transport. However the use of surface water can still substantially be improved.

The availability of salt water enables salt production, deep sea fishing and shell fish and shrimp production. These economic sectors can still be improved.

The coastal location of Bangladesh has also led to the development of relatively big sea ports that can still be better connected to inland ports.

Particularly the sea ports have also attracted major marine industries like shipbuilding and Bangladesh is also known for its ship breaking experience, bringing iron and other raw materials into the country. GoB is actively supporting these activities via its industrial and land use zoning policies.

However, there are some worrying developments occurring in the river systems of Bangladesh. Due to upstream developments that lead to increasing off take of water and under the influence of climate change, the periods with critical low flow conditions of rivers are already increasing and they are likely to increase further. On the other hand it is also expected that climate change will also lead to increased peak flows. These changes in river flow conditions have many consequences for water management such as increased salinity intrusion in coastal areas and higher risk of riverbank erosion. Saline water intrusion is highly seasonal in Bangladesh and the seasonal variations are dominant factors for coastal eco-systems, fisheries and aquaculture as well as agriculture. Therefore, any changes in the present spatial and temporal variation of salinity will affect the biophysical system of the coastal area and is making existing water supply sources and freshwater ecosystems vulnerable.

Map with the salt intrusion front. (attached)

Riverbank and island erosion is a major water management issue. Although the annual loss of land due to erosion decreased from about 10,000 hectares per annum to present levels of around 6,000 ha per year; in some parts of the country erosion is a bigger problem than flooding. On the other hand there is also land accretion. The average annual pace of net accretion in the Meghna Estuary was about 18.8 km2 over the last 30 years.

With respect to water and biodiversity, especially the mangrove forests (the Sundarbans) along the coast are highly valuable, also as a first barrier against tidal and storm surges. However, this unique ecosystem with the endangered Royal Bengali Tigers in it, is also under high pressure from encroachment and exploitation. The area is also vulnerable to lower river inflows due to increasing upstream off take of river water, climate change and sea level rise.

In general the management, including maintenance, of the water resources and their use as well as the construction, operation and maintenance of water infrastructure, continue to receive too little attention and funding. Due to the above mentioned demographic, economic and climate changes, proper management of the water resources and their utilization is essential to support the goal of sustainable economic growth in Bangladesh.

In a delta country with a highly complex water situation, it is very important to combine flood control, drainage and irrigation, especially in coastal areas where both fresh and salt water has to be managed. Besides it is important to look at both surface water and ground water in an integrated way.

The over pumping of aquifers for drinking water in urban areas like Dhaka, where the water table is going down with about 4 meters every year is a case in point, also because it will lead to submerging land, making cities more vulnerable to flooding. Over pumping of water for irrigation is another potential danger, which can also accelerate salt intrusion.

Review of past performances of the water sector

The water resources sector is undergoing significant shifts in terms of policies, strategies, plans and programs. The Water Resources Planning Act (1992), the National Water Policy (NWPo-1999), the National Shipping Policy (2000), the National Water Management Plan (NWMP-2001), the Guidelines for Participatory Water Management (GPWM-2000), the Coastal Zone Policy (CZPo-2005),  the Coastal Development Strategy (CDS-2006), the Bangladesh Water Development Board Act (2000), and the National Water Act (draft) have created an enabling environment in the country to practice Integrated Water Recourse Management (IWRM) for economic, social and environmental sustainability. Besides, GoB has recently created more space for private sector involvement in the water sector, so far especially in dredging, inland water transport, waste water treatment and water based tourism. It is envisaged that other components of the water sector will follow suit, although government and Water Management Organizations will have to play a major role in creating and maintaining strategic water infrastructure, also in the future. The programs and projects of the water sector in the Sixth Five Year Plan will thus be implemented in this favorable policy environment.

The initiatives of the Local Government Engineering Department (LGED), supporting the development of Small Scale Water Resources (SSWR) (less than 1,000 hectare) started in

1960 under the Thana Irrigation Program. During 1986 to 1996, 60 SSWR schemes were implemented under the Rural Employment Sector Program. Taking lessons from the performances of the earlier water resources development projects, sustainable use of water resources has been facilitated with the participation of local stakeholders along with local government institutions, involving the public and private sectors, communities and individuals in implementation of the Small Scale Water Resources Development Sector Projects (SSW-1 and SSW-2). Both SSW-1 and SSW-2 have enhanced rural incomes by developing community based Water Management Organizations (WMOs) and community managed small scale infrastructure that enabled more than one cropping season per year. This approach contributed to the successful drive to reduce rural poverty in Bangladesh and to improve the food security situation of the country.

Up to June 2010, about 5.90 million hectares of flood vulnerable land have been brought under flood control and many drainage improvement facilities have been created by the Bangladesh Water Development Board. However, financial resources were generally limited. This has led to suboptimal size and/or quality of both sea and inland embankments in various parts of the country, making resectioning of dikes a necessity.

By June 2010 the BWDB had also provided irrigation to about 1.40 million hectares of land under surface water irrigation projects. In addition, about 0.10 million hectares of land had been reclaimed from the Bay of Bengal, creating space for settlement of poor landless people.

The direct impacts of a total of about 735 projects implemented so far by BWDB are: (i) the creation of a more secure environment for crop production by preventing excessive flooding in normal years and providing opportunities for multiple harvests instead of only one per year, (ii) generation of new rural employment in construction and maintenance of embankments and other water infrastructure, (iii) protection of agricultural land and buildings against river erosion and (iv) reclamation of land.

The indirect benefits of the BWDB projects are (a) better communication via roads on embankments; (b) more security from water-borne hazards, (c) an increased primary defense against sea level rise and (d) enhancement of agro-based economic activities in a flood-free, secured environment. Particularly regarding the latter, BWDB also facilitated that Water Management Organizations started to act as catalysts for various services from other government agencies as well as NGOs, e.g. from the Departments of Agricultural Extensions, Livestock and Fisheries, etc. Some WMOs also have contacts with private sector players. An important effect of the improved flood control has been that DAE could successfully promote the use of High Yielding Varieties of rice that need an environment without excessive flooding.

Irrigation is one of the most important and essential parts for increased agricultural production and thus farm productivity and agricultural growth. That is why the Government has given priority to irrigation for increased agricultural production. It has been stated that the use of irrigation should also be expanded into the Monga prone areas and other less productive areas. This can also help in reducing regional disparities in wealth.

Especially when ground water is used for irrigation careful planning of water use is needed to prevent over pumping of aquifers and falling water tables.

Government has therefore also given importance to the effective and optimum utilization of surface water rather than extracting too much ground water for irrigation.

By preserving some of the available surface water from the monsoon period, e.g. in canals, this water can partially be used in the dry season for irrigation. This can also   help to recharge the ground water. Government is also promoting higher water use efficiency, particularly in agriculture, the economic sector with the highest water use figures.

Recently government also revived its efforts to develop the river systems, via dredging, river training and river bank protection. Measures are also taken to limit water pollution.

GoB also stimulates the private sector to get involved in the inland water transport logistics in order to make these more efficient and competitive. Domestic and foreign private sector investments are now also used to construct Inland Container Terminals and other major water transport related infrastructure. The government has also entered into negotiations with neighboring countries, particularly India, to develop rail-river corridors through Bangladesh for regional transit freight traffic, e.g. between different states of India and to a lesser extend to Nepal and Bhutan.

Vision towards a national delta plan/strategy

Economic growth, employment creation and poverty alleviation are guiding objectives for the development strategies of the Sixth Five Year Plan, for which interventions in the water management sphere need to create the pre-conditions.

The benefits derived from water sector interventions are contributing to the economic growth, poverty reduction and food security initiatives of the country. Through Flood Control & Drainage projects and to a lesser extend also irrigation schemes, BWDB assists to create a favorable environment for the use of HYVs in the production of rice and other crops. This induces enhanced employment generation for rural agricultural laborers. Moreover, construction works and annual operation and maintenance works related to water infrastructure provide wage income opportunities for the rural skilled and unskilled laborers that are in monetary terms roughly equivalent to one-third of the total annual agricultural wage income. This can be increased when more kilometers of sea and inland embankments are upgraded, using labor intensive construction methods. The water related labor intensive public works in rural areas can contribute to social stability in the countryside during the transition to more commercial agricultural production systems with involvement of the corporate private sector, especially in the agro-processing and distribution parts of value chains, which often lead to various forms of contract farming by the most progressive farmers in the rural areas, who then gradually enlarge their farms. Embankment construction and canal re-excavation can absorb some of the resulting excess labor of landless households.

The water sector intends to bring relevant government structures, knowledge and capacity building institutes, the private businesses sector and non-governmental organizations together in order to develop a shared vision on an incremental national delta development process. To get the necessary political, bureaucratic, academic and civic support, this process will have to be participatory and infused by science.

The need for integrated water management in order to reach sustainable delta development can be visualized by the framework for delta development and planning, used by the (global) Delta Alliance (2010).

In this natural resources based conceptual framework that is emphasizing water resources, three interrelated layers are distinguished (see the figure below): the users system, the infrastructure system and the natural system. The users system is where the economic activities take place: agriculture, fisheries and aquaculture, (water) transport, etc. It poses demands to the natural system, such as water resources. Wherever this demand cannot be supplied directly by the natural system, water infrastructure is created. This infrastructure consists of embankments, regulators, irrigation and drainage channels, etc.

Each of these three systems is influenced by different drivers of change. For instance, the demands on the water resources are driven mainly by economic and demographic developments, whereas the supply of water resources (natural system) is predominantly influenced by external drivers, such as climate change, upstream (trans-boundary) water off-take and (local) subsidence. Ultimately the management of these three interrelated systems should be a governance system that is also dedicated to influence these three system layers and their interactions via policy measures. GoB is aiming to do just that.

User system

agriculture

fisheries

transport, etc.

Infrastructure

sea and inland embankments

regulators, sluices

controlled flooding

irrigation  & drainage systems

barrages, etc.

Natural system

rivers

estuaries/ coasts

salinity front

(mangrove) forests, etc.

Governance

Legislation and regulation

public investments

public private partnerships

Stakeholder participation

Water management Organizations

Communication and information

Etc.

Climate change

Subsidence

Trans-boundary flows

Demographic trends

Economic developments

Technological developments

Figure: Framework for delta development and planning (Delta Alliance, 2010)

The aim of integrated water management is an effective and efficient integration of all relevant sectors to guide societies towards sustainable development. Water is a cross-cutting sector. Water provides an enabling environment for human development. But it can also be a hazard.

In order to make optimal use of the existing water resources in Bangladesh and to anticipate on the water hazards there are three basic management keys one can turn for each of the three layers:

- User system layer: adaptations of the water use systems (demand management)

- Infrastructure layer: investments / improvements in water infrastructure

- Natural system layer: protection / rehabilitation of the natural system

Key Water development strategies

1. Sustainable River Development

Rivers perform major functions for life in Bangladesh. They provide water and often fertile sediment for crop and fish production, drinking water and water for industrial and domestic purposes. The rivers also form an important transport network. Furthermore, they have shaped Bangladesh as a delta country through a dynamic interplay with marine influences whereby over the past millennia erosion materials from the Himalayas and other parts of upstream countries were deposited as sediments, especially in the lowest parts of Bangladesh.

Due to their natural behavior as freely meandering and braiding rivers, the rivers also cause adverse conditions through river bank erosion and extreme flooding, which is becoming a more important problem now that the economy of Bangladesh is going through a transition from a primarily agriculture based economy to a more industry and services oriented economy. The fact that rivers become shallow and extremely wide is also limiting the transport of bulk goods over water, leading to higher prices when they have to be transported by rail or road. It also makes infrastructure like bridges more expensive.

A sustainable river development strategy is aimed at mitigating the adverse impacts of rivers while maintaining their benefits to society. One of the guiding principles to this strategy is that interventions do not aim at completely harnessing river processes, but instead steer them into the right direction. An example is the renewed attention towards enhancing the conveyance capacity of rivers through dredging and river training that can also make the rivers narrower. Another example is the reduction of flood vulnerability through integrated flood risk management instead of relying on flood protection only.

Of course, in many situations an upgrade and expansion of the existing water infrastructure (embankments, culverts, canals, dams, etc.) is of major importance to improve the current situation. But this will be combined with other, non-engineering measures that make use of the entire spectrum of measures, ranging from innovative water use adaptations to eco-engineering and novel governance approaches.

River Dredging/ River Training and Prevention of River Bank Erosion

Besides flash flood hazards, erosion of river banks and islands in rivers pose major problems for the living conditions of millions of people living in or close to rivers in Bangladesh. Since this type of erosion originates from the natural behavior of lowland rivers to meander, i.e. to move its bends sideways, the problem is not easy to solve. Hard engineering solutions such as revetments and rip-rap are often an expensive and unsustainable solution to stop the erosion. The implementation of smart, science based river dredging methods in combination with innovative river training methods (e.g. using semi-permeable structures and bottom-vanes) could provide effective alternatives. Although dredging is less visible than pouring concrete and it needs to be repeated, the results are often better especially when it is combined with river training methods. Smart dredging and river training can steer the natural erosion process to where it is less damaging.

Environmental flows / trans-boundary cooperation

Due to upstream water resources projects and developments, particularly in India, the dry season flow in rivers of Bangladesh is decreasing. Sufficient dry season flow is necessary for meeting downstream economic and ecological requirements such as navigation, fisheries and aquaculture, agriculture, prevention of salinity intrusion, maintenance of the assimilative capacity of the rivers (during dry seasons the river water quality is likely to deteriorate) as well as proper ecological functioning of the wetlands, estuaries and floodplains.

River flow is also vital for maintaining sufficient depth for inland vessels and for the spawning of different fish and other aquatic species. Besides, river water flow above a threshold level is necessary to limit the salinity intrusion near the Bay of Bengal, which is developing into a serious problem. Particularly in the Southwestern region of Bangladesh decreasing river flow has already caused the salinity level to rise considerably, causing adverse impacts to the corresponding areas. Assessment of environmental flow requirements in Bangladesh has become urgent for several rivers that are already experiencing problems with their dry season flows. Examples are the Gorai river (Southwest), Teesta river (Northwest) and Surma-Kushiyara rivers (Northeast). Since many of the rivers flowing through Bangladesh originate in other countries, setting environmental flow conditions is a very important component in trans-boundary cooperation. Assessing the minimum water needs is therefore a vital instrument in negotiations with other countries. Besides the country of course has to ensure and prove that it is using fresh water resources effectively and efficiently to convince neighboring countries that it needs the water.

2. Integrated Coastal Zone Management (ICZM)

The Government of Bangladesh has formulated a Coastal Zone Policy which was approved by the government on 17 January, 2005. The Policy provides a general guidance to all concerned for the development and management of the coastal zone in a manner so that the coastal people are able to pursue their lives and livelihoods within a secure and conducive environment. There were three reasons for initiating this Coastal Zone Policy:

- The coastal zone is lagging behind in socio-economic developments;

- Initiatives to cope with different disasters and with the gradual deterioration of the environment are limited;

- The coastal zone has the potential to contribute much to the national development.

The Coastal Development Strategy (CDS) was formulated and approved at the 2nd meeting of the Inter-Ministerial Steering Committee on ICZMP held on February 13, 2006. This Coastal Development Strategy focuses on the implementation of the Coastal Zone Policy. The CDS distinguishes nine strategic priorities, evolved through a consultation process that can guide interventions and investments in the coastal zone.

Ensuring fresh and safe water availability

Salinity intrusion and arsenic contamination provide significant constraints in the supply of fresh water for human consumption, agriculture and industrial development. The only reliable sources of fresh water in the coastal areas are the deep-water aquifers, which are under high risk of being ‘mined’ or becoming infiltrated with salt or arsenic contaminated water. Improving the management of groundwater and providing safe water supply to the inhabitants of the coastal zone is therefore of prime priority. Furthermore, increasing the freshwater flow from the rivers (esp. the Gorai) towards the Southern coastal part of Bangladesh is considered a major measure to push back the salinity front, to recharge aquifers and to sustain water supply for domestic use and irrigation.

Safety from man-made and natural hazards

In view of recurrent tidal and storm surges and the expected climate change induced sea level rise, the strengthening and rehabilitation of sea dikes is an important and necessary step. Currently the sea embankments are only calculated on the basis of the daily tidal movements, not to protect land against tidal and storm surges. GoB will therefore work on resectioning of sea embankments in the coming five years. This is however not the only measure that needs to be taken to increase safety from salt water floods. In line with the principles of integrated flood risk management also the reduction of coastal vulnerability through the construction of multi-purpose cyclone shelters is an important component. Furthermore, the use of so-called “building with nature” concepts, such as protection against erosion through mangroves and oyster beds needs to be enhanced.

Optimizing use of coastal lands

Pressure on space is high in the coastal area and is expected to increase. Therefore land reclamation can offer a new dimension of poverty alleviation by allocating part of the new lands to landless people.

Promoting economic growth, emphasizing non-farm rural employment

Economic sectors such as tourism, water transport and ports and (agro-)industries are important to provide new jobs and increased income to the coastal population.

Sustainable management of natural resources

Capture fisheries and aquaculture are important income earning activities. Overfishing and intensive aquaculture practices, however, often result in unsustainable development and a loss of both environmental quality and local income. Initiatives towards socially responsive and environmentally friendly fresh and salt water shrimp farming and sustainable river and sea fisheries remain of crucial importance for a more sustainable management of natural resources.

Improving livelihood conditions of people (especially women), environmental conservation, empowerment through knowledge management and creating an enabling institutional environment are other strategic priorities..

The integrated coastal zone management approach is partially already under implementation in an incremental way, under a series of programs and projects supporting IWRM and the development of WMOs in the Southwest, South central and Southeast zones of BDWB. Besides, several national level programs work along similar lines. These activities are gradually developing more linkages with each other, leading  to the necessary ‘ critical mass ‘ and to harmonization of approaches, leaving enough space for necessary adjustments at certain specific localities and flexibility to pick up useful local innovations and up scale these, particularly in the field of (community based) climate change adaptation.

3. Towards a Bangladesh Delta Plan 2100

In the coming five years GoB also intends to work towards a comprehensive National Delta Plan 2100 to guide the country towards accelerated and sustainable development in the framework of durable use of natural resources, particularly water.

The Delta Plan will combine integrated water resources management with spatial planning, including land use zoning, and local economic development policies and strategies.

In the process to develop a clear Delta Vision for the country, the Ministry of Water Resources and the cluster of institutions around it, the Ministry of Local Government Rural Development and Cooperatives, the Ministry of Agriculture and the Ministry of Environment & Forests will have to play key roles, next to the Ministry of Land, the Ministry of Planning and other relevant ministries. GoB also realises that for a common Delta Development Vision a so called GKBN approach is needed. Not only politicians and civil servants within government need to gradually develop a shared vision on the envisaged future water situation in the country, taking climate change into consideration, but also knowledge institutes, businesses and NGO’s. GoB considers this of great importance because it realises that fragmented and contradictory approaches to water management and land use zoning can lead to conflicts and hazardous situations.

GoB envisages that the Bangladesh Delta Plan 2100 will be in place and widely disseminated by the end of 2015, forming a solid spatial foundation for long term planning. It also envisages that the Delta Plan 2100 is a living document that is already under implementation in 2015 in an incremental way in several parts of the country that are of great significance in terms of water management. Improved coordination between existing water related activities is considered an important strategy element of the upcoming Delta Plan. This can lead to resolving contradictory issues, creating more synergy between various stakeholders and full integration of climate change in both water management and land use zoning as well as economic policies.

The Bangladesh National Delta Plan will be developed by a broad range of actors and in cooperation with other delta countries, both within the South East Asia region and elsewhere. In the planning process several so called ‘hotspots‘ will be chosen based on two basic criteria: their economic development potential and the complexity and vulnerability of their water situation, also in the light of climate change. The Southern coastal area and the haor area, but also Dhaka and its surroundings where the pollution of rivers needs urgent attention are examples of such potential hotspots. The Delta Plan will develop a vision on how a healthy living and working environment can be ensured and sustainable economic development can be created with special reference to the consequences of climate change on the water sector.

The role of chosen ‘hotspots’ is also to illustrate how integrated water management in a specific area can be implemented in practice, based on spatial planning, land use zoning, water development and management and local economic development policies. In the hotspots it will be shown how relevant policy sectors and government agencies, knowledge institutions, private businesses, CBO’s and NGO’s can effectively work together.

Multi-hazard / risk management is another component of IWRM that will be given attention in the context of the delta planning process.

4. Water Governance and roles of local WMO’s

Based on earlier experiences GoB has decided several years ago to opt for science infused participatory water management in rural areas, making local Water Management Organisations (WMOs) that are covering hydrological units, the backbone of the national water management system.WMOs have the legal status of multipurpose cooperatives and can

build up financial reserves to cover O&M costs of water infrastructure. WMOs are mandated to protect all people living within a hydrological unit against excessive floods and to ensure the availability of drinking water for everybody and where possible also for irrigated crop production, (fresh water) aquaculture and other economic activities. They are responsible for operation and maintenance of minor water infrastructure like inland embankments, regulators, drainage and irrigation channels, etc. and for managing the water and waste water flows, accelerating drainage, manage fresh water buffers, keeping salty water out when needed, etc.

The Local Government Engineering Department is supporting the WMOs in smaller (less than 1,000 ha) hydrological units, while the BWDB provides technical guidance to local WMOs that are operating in areas above 1,000 hectares. The BWDB and the LGED also assist these WMO’s in managerial tasks like (preparing for) levying and collecting service charges for Operation & Maintenance (O&M) and to work towards cost recovery of Flood Control, Drainage, Irrigation and Aquaculture projects. WMOs generally also maintain working relationships with the Local Government Institutions like districts, sub-districts (upazillas) and unions, which are covering political administrative areas, that are overlapping with the WMOs’ hydrological units. They also work together with various deconcentrated government agencies and with (I)NGOs and sometimes also with external private sector businesses.

Some WMOs already charge water use fees for irrigation and aquaculture. Part of the labour for e.g. embankment rehabilitation is provided as voluntary labour by WMO members. Contractors and Landless Contracting Societies (LCSs) are supervised by WMO members who have been trained by BWDB or LGED. In principle, WMOs do not only represent main water users like farmers and people involved in aquaculture, but also poorer segments of the population within a hydrological unit, including fishermen and destitute female heads of households. WMOs are mandated to resolve conflicts between water users, including the often difficult conflicts between shrimp growers that want salt water and farmers that want fresh water to irrigate their crops. In practice strong WMOs also act as catalysts for various services, ranging from agricultural extension, consulting WMOs to choose farmers who could participate in Integrated Pest Management field tests or in field trials for new crop varieties, etc. to livestock and fisheries extension and introduction of biogas slurry as a compost enhancing option. Here and there part of the WMOs’ O&M funds are lended on to poor households that want to build up or expand their asset base, e.g. via poultry or vegetable gardens.

There are about 7,850 WMOs in the BWDB data base right now. However their managerial and technical capacity still varies enormously. To cover the whole country with WMOs it has been calculated that about 55,000 will be needed. However, when areas with complex water situations are prioritised, this number will be substantially less. It has been recommended to group WMOs as much as possible within the framework of the about 160 bigger water catchment areas in the country.

In the coming five years, GoB intends to improve the capacity of prioritised WMOs in the so called ‘hotspots‘ of the National Delta Plan 2100, that have great economic potential but that are also vulnerable to climate change effects and other water related processes like the increasing number of dams upstream in the joint Ganges-Padma, Brahmaputra-Jamuna and the Meghna river basins. This will mean that special attention will have to be given to WMOs in e.g. the Southern sub delta/ coastal zone and the haor wetland area in the North East.

Besides, GoB aims to realise a substantial expansion of WMOs, mainly in those areas, e.g. by facilitating horizontal exchange between strong and weaker WMOs, by enabling the strongest WMOs to become Learning Centres for others, by brokering training opportunities for WMO-members e.g. within local government and bookkeeping training institutions, by establishing a special Training of Trainers centre for key members of the strongest WMOs, that is focussing on technical and managerial issues that are specific for WMOs.

The Integrated Planning for Sustainable Water Management (IPSWAM) program that is centrally located in the Southern coastal zones and that has developed the by now broadly accepted six step methodology for IWRM and WMO development, is already used as the main hub for ‘on the job training‘ in a network of WMOs that receive support from BWDB and LGED. This program will be expanded in the coming five years and GoB will enable it to establish more linkages with other government agencies as well as other actors, including (Unions of) Water Management Organizations in other countries, to speed up capacity building within WMOs and to assist in finding creative solutions to new environmental challenges posed by climate change.

Regarding the position of WMOs vis-à-vis other government institutions, GoB has taken the decision to make it mandatory for all government agencies to work closely together with the WMOs. In practice this is already happening in areas with strong WMOs but this new government policy intends to support efforts to up scale the number of well- functioning WMOs by enabling their position to broker contacts with various services within hydrological units, which can raise their social status.

5. Climate Change Adaptation

Due to its geophysical properties (e.g. low elevation in a major delta), Bangladesh is considered as one of the most vulnerable countries in the world regarding climate variability and climate change. It already experiences more frequent storm surges in the coastal areas and the changing rainfall patterns are expected to lead to more droughts on the one hand and more floods during the monsoon, which also makes Bangladesh one of the most disaster prone areas in the world. While in normal years about 20% of the country is inundated by floods this can go up to 60% in extreme years. Strengthening the resilience to climate change is pivotal in all development and poverty alleviation activities in the country. An integrated approach to incorporate climate change adaptation will be required.

Due to population growth and economic development there is also an urgent need to further improve food production which will lead to further increasing use of water for irigation. Also fresh water demands from households and industry are likely to increase in the future. The increasing urbanization and continuing population growth add pressure on the water resources that are becoming scarce, especially in the dry season. Although climate change will have contradicting effects, it is likely that it will make the water situation in the country more problematic. Due to climate change the dry season water supply from the major river systems will become more unreliable and droughts could occur more frequently and become more severe. Due to sea level rise and changing cyclone risks also flood and disaster prevention and management need to adapt to the changing circumstances. Climate change will also affect water quality through salt water intrusion. In addition, low river flows will cause an increase in the pollutant concentration when the pollution load remains the same. This affects fish and agricultural production systems.

Improving the capacity to manage water related climate change effects

Bangladesh is internationally in the forefront of climate change thinking and negotiations; however the available information at the national level and the capacity to manage climate change effects is still limited. In general the population has a high resilience level and there are promising examples of - often community led - adaptation. These can serve as inspiration nodes for both people within the country and for foreigners. In the short term there is a need for capacity development in relation to innovative climate change adaptation at different levels. Universities and colleges will have to develop climate change adaptation courses and new action research lines, preferably together with practitioners in implementing organisations and businesses. Options like combining oyster production with breaking the power of waves in the sea or using rainwater harvesting to replenish saline aquifers with fresh water, are examples of new water related technologies that can be explored. Also the knowledge level of policy makers and civil servants at national and local level needs to be raised through e.g. structured field visits, documentaries, newsletters, short courses, seminars, cultural activities, exhibitions, etc. Parliament, the media and our film makers, actors, musicians, photographers, artists, etc. will also have to play a role. Most of the above mentioned activities can be implemented in the framework of the envisaged National Delta Plan 2100.

Reducing and managing water and climate change uncertainties

There is still a lot of uncertainty about the possible impacts of water related climate change on Bangladesh. These uncertainties can be reduced by improved research and information systems. There is therefore a need for more and better water and climate change impact studies and especially in providing information in accessible formats that can facilitate decision making and that can provide user friendly information to various media and networks of e.g. WMOs and other relevant grassroots organizations. It is also important that the water sector aims to better manage uncertainties and improve its adaptive capacity.

Combining soft and hard adaptation measures

One way of managing uncertainties is developing a set of adaptation measures which combine technical and non-technical interventions. For example, drought vulnerability can be reduced through fresh water storage and irrigation techniques in combination with improved insurance schemes for harvests. Flood vulnerability in the Southern part of the country can be reduced by combining an improved coastal protection system with more flood resilient housing and better flood warning systems. Also in the Haor area and the central part of the country the water security situation can be improved with both types of measures. GoB intends to encourage and facilitate this type of approaches in the field of water and climate change.

Improving the adaptive capacity of poor segments of the population

Many studies have shown that all over the world poor people are most vulnerable to water related climate change. They often live in regions with a low productivity, which are most vulnerable to droughts. Also poor people tend to live in flood prone areas. In addition, the poor often lack the capacity to invest in infrastructural measures to adapt to climate change and to respond adequately to disasters. Climate change adaptation strategies therefore will have to take the needs of poor households into account, supporting and complementing their own initiatives in dealing with climate variability and change, often in close cooperation with Water Management Organizations.

6. Development of a water and climate services system

To further develop Bangladesh as a delta country and to efficiently adapt to climate variability and change there is a need to develop reliable and trusted information systems. Climate has a great impact on water resources management at different time scales.

The information coming from climate science and weather services is not always directly useful for water management. GoB therefore aims to develop a Water & Climate Services System to provide information to both the private and public sector on water availability and quality, on drought and flood risks, and on other relevant water and climate issues. This information system could also become the scientific data base for the envisaged Bangladesh National Delta Plan 2100.

In this Water & Climate Services System satellites deliver information from space which

- after translation and interpretation - can be used to decide which levies and dikes need to be improved and which river systems can be used for navigation. The system can also supply agricultural yield predictions. Satellite information can also be coupled with hydrological models to predict floods and droughts and to supply information on how much water for irrigation will be needed in different parts of the country.

The coupling of hydrological models, with climate and spatial information can supply the necessary long term information on the impacts of sea level rise and river water off take levels on for example salt water intrusion and flood risks. It can also be used to guide spatial planning and economic development policies. For example clusters of high value developments should preferably be guided towards areas with a low flood risk and water demanding industries should in principle be planned in areas where fresh water availability is more or less guaranteed.

The envisaged Water & Climate information system can also serve to manage floods and to reduce flood risks. It can also be used as an improved flood early warning system. It can supply the necessary information to decide where improved flood protection is needed through the development or resectioning of dikes, levees and improved drainage systems.

The Water & Climate information system can only function optimally if different institutes collaborate and when there are clear protocols for information sharing between different regions, institutions and organizations. GoB aims to develop a well functioning Water and Climate Services System, with operational information sharing protocols.

SFYP Objectives and Targets for Water Resources Management

The objectives of the water resources sector for the SFYP can be grouped according to the global framework for delta planning:

Generic:

- Food security, economic growth, poverty alleviation and employment generation,

- Climate change adaption and mitigation,

- Integrated coastal zone management,

Adaptations of the natural resources users system (demand management)

- Land use zoning and conjunctive and efficient use of surface and groundwater for sustainable irrigation and fresh water aquaculture where possible, halting the salinity frontier,

- Optimum use of available flows of the common rivers for multipurpose use,

- Water conservation for irrigation and other uses in both reservoirs and subsurface in aquifers,

- More productive and more sustainable agriculture and (cultured) fisheries, especially in the completed project areas of BWDB and LGED where WMOs have been established,

Investments / improvements in water infrastructure

- Protection against the sea and against river erosion,

- Land reclamation,

- Enhancing the conveyance capacity of water courses through river dredging and river training,

- Flood Control/ Flood Management,

- Ensuring year-round sustainable agriculture and aquaculture via (complementary) irrigation and drainage systems,

Protection / rehabilitation of the natural system

- Environmental protection,

- Prevention of saline intrusion through augmenting the fresh water flow into the South, particularly the Southwestern part of the coastal zone including the brackish Sundarbans with the mangrove forest (a  world heritage site),

Institutional and governance measures

- People’s participation ensured in conformity with Integrated Water Resources Management (IWRM) principals and the number of well functioning Water Management Organizations increased substantially, which is measured by using a detailed baseline data base of WMOs,

- Regional and International cooperation for basin-wide water resources development and management of trans-boundary rivers enhanced,

- Water resources management institutions are strengthened and have more capacity in the fields of climate change adaptation, water and climate change data management and

analysis, management of rivers and coastal areas, and they are at many levels better connected to the digital world,

- Studies and research on future water resources management have successfully been conducted and have led to adjustments in practice and policies.

The following results and proxy indicators have been chosen for inclusion in the SFYP results framework from a water management perspective:

| Outcomes for Water Management | Indicators | Baseline (latest Available) | Targets 2011/ 2015 | Responsible Agencies (Gov and DP) |
| --- | --- | --- | --- | --- |
| The most strategic river systems are better functioning from an economic and environmental perspective. | No. of kilometers of waterways navigable year round. | Monsoon: 5,200 km; Dry season: 3,800 km. | 2011: Additional 10 km; 2015: Additional 100 km | GoB: BWDB; MoShip |
| Increased protection of productive land against the sea. | No. of kilometers resectioned coastal embankment, making them higher and stronger. | Existing 4,841 km of coastal embankments | 2011: additional 40 km; 2015: additional 480 km | GoB: Ministry of Water Resources (BWDB; IWM; CEGIS), |
| Increased human and institutional capacity to manage water resources. | No. of Water Management Organizations. | 7,898 | 2011: additional 100 2015: additional 3,000 | GoB: MoWR (especially BWDB) MoLGRDC (especially LGED); |

| More sustainable and efficient water use in irrigation | Reduced rate of groundwater depletion |  | 2011: 5% 2015: 50% |  |
| --- | --- | --- | --- | --- |

In addition the following more detailed targets have been set:

Table: SFYP targets of water management sector

| Issues |  |
| --- | --- |
| Expected Outcome |  |
| Employment Generation | 12.5 millions days |
| Poverty Alleviation | Water projects often involve poor people |
| Protection of the Environment | EIAs are mandatory for all water projects |
| Social Security | Many water projects are pro-poor |
| Food Security | Especially FCDI projects contribute |
| Flood Control | 0.7 million ha. |
| Human Resources Development | Training of 30.000 persons, especially WMO members. |
| Outputs |  |
| Dredging of rivers | 318 km. |
| River bank protection (New) | 158 km. |
| River bank protection (Rehabilitation) | 142 km. |
| Inland embankments (New) | 690 km. |
| Resectioning of inland embankments | 469 km. |
| Coastal embankments (New) | 45 km. |
| Resectioning of coastal embankments | 480 km. |
| Coastal cross-dams | 19 nos |
| Excavation of irrigation canals (New) | 1.067 km. |
| Re-excavation of irrigation canals | 1.124 km. |
| (Re-)Excavation of drainage channels | 636 km. |
| Hydraulic structures (regulators, sluices) | 1.117 nos |
| Rehabilitation of irrigation structures | 440 nos |
| Construction/Rehabilitation FC structures | 365 nos |
| Reservoirs | 2 nos |
| Rubber dams | 6 nos |
| Barrages | 2 nos |
| Installation pump houses & rehabilitation | 9 nos |
| New Water Management Associations formed with clusters of WMOs | 10 nos in water catchment areas and existing WMOs and WMAs strengthened |

References

GoB policy documents.

Bucx, T., M. Marchand, A. Makaske, C. van de Guchte (2010). Comparative assessment of the vulnerability and resilience of 10 deltas – synthesis report. Delta Alliance Report number 1. Delta Alliance International, Delft – Wageningen, the Netherlands.